UNIVERSITY OF WATERLOO AQ
Faculty of Engineering 
      Department of Electrical and Computer Engineering       
ECE 621- Computer Organization

Register File and Execute Stage

Group 2

Jordan Ross, Mustafa Faraj 
j25ross, mabdulaz 
31 October, 2014

# ALU Module Code

This section contains the code for our ALU module. It takes in the two source operands as well as an operation control signal (set in the decode stage). It outputs the result of the arithmetic operation as well as set two condition bits (zero and negative).

module alu(

input [31:0] op1, // operand 1 (always from rs)

input [31:0] op2, // operand 2

input [5:0] operation, // The arithmatic operation to perform

input [5:0] shift_amount, // The number of bits to shift

output reg [31:0] result, // The arithmatic result based on the operation

output reg zero, // Indicates if the result of the operation is zero.

output reg neg // Indicates if the result is negative or not.

);

// The operations that the ALU supports are:

// 0: Addition TODO: Is there a difference here with unisgned vs. signed?

// 1: Subtraction

// 2: Multiply

// 3: Divide

// 4: Shift logical left

// 5: Shift logical right

// 6: Set on less than

// 7: And

// 8: Or

// 9: Xor

// 10: Nor

// 11: SRA

// 12: Load upper

always @(op1 or op2 or operation or shift_amount) begin

case (operation)

0: result = op1+op2;

1: result = op1-op2;

2: result = op1*op2;

3: begin

if (op2 != 0) begin

result = op1/op2;

end

// TODO: Otherwise we should trap here

end

4: result = op2 << shift_amount;

5: result = op2 >> shift_amount;

6: result = op1 < op2 ? 1 : 0;

7: result = op1 & op2;

8: result = op1 | op2;

9: result = op1 ^ op2;

10: result = ~(op1 | op2);

11: result = op2 >>> shift_amount;

12: result = (op2 << 16)&32'hffff0000;

endcase

zero = result == 0 ? 1 : 0;

neg = result[31];

end

endmodule

# Branch Resolve Unit

This section contains our logic for selecting whether a branch should be taken or not based on the two condition bits set by the ALU.

module branch_resolve(

input zero,

input neg,

input [1:0]branch_type,

input is_branch,

output reg branch_taken // Indicates if the branch is taken or not

);

always @(zero or neg or branch_type or is_branch) begin

if (is_branch == 1'b1) begin

case (branch_type)

0: branch_taken = zero == 0 ? 1 : 0; // BEQ

1: branch_taken = zero != 0 ? 1 : 0; // BNE

2: branch_taken = (neg | zero) == 0 ? 1 : 0; // BLEZ

3: branch_taken = neg != 0 ? 1 : 0; // BGTZ

endcase

end else begin

branch_taken = 0;

end

end

endmodule

# Processor Module Code

This section contains the code for the processor module. The processor module is our top level module where we instantiate our fetch, decode, memory, register file, and ALU so far. This is also where we have our control signals being set. The control is only being done in the decode stage and the control signals that are needed in later stages are latched in pipeline registers. The processor module currently has the IF/ID and ID/IX pipeline registers implemented.

module processor(

input clk,  // The system clock

input srec_parse    // If the SREC parser is active or not.

);

// Decoder signals

wire [4:0]rs;

wire [4:0]rt;

wire [4:0]rd;

wire [4:0]sha;

wire [5:0]func;

wire [15:0]immed;

wire [25:0]target;

wire [5:0]opcode;

wire [31:0]pc_out;

wire [31:0]insn_out;

// Control signals

reg stall;

wire [1:0]insn_access_size;

wire [1:0]fetch_access_size;

wire fetch_rw;

wire insn_rw;

reg reg_file_write_enable;

// Address lines

wire [31:0]pc;

// Data lines

wire [31:0]insn_data_out;

wire [31:0]insn_address;

// SREC registers (only used for helping the parser write to instruction memory)

reg [31:0]srec_address;

reg [31:0]srec_data_in;

reg srec_rw;

reg [1:0]srec_access_size;

// Decode wires

wire [31:0]decode_pc; // We define this as the PC for next instruction to be executed

wire [31:0]decode_ir; // We define this as the current instruction being decoded

wire [31:0]dec_A;

wire [31:0]dec_B;

wire [4:0]dest_reg;

// Decode control signals

reg dest_reg_sel;

reg dec_illegal_insn;

reg [5:0] dec_alu_op;

reg dec_is_branch;

reg dec_op2_sel;

reg [5:0] dec_shift_amount;

reg [1:0]dec_branch_type; // 0-BEQ, 1-BNE, 2-BLEZ, 3-BGTZ

reg dec_is_jump;

// Execute wires

wire [31:0]exe_pc; // We define this as the PC for next instruction to be executed

wire [31:0]exe_ir; // We define this as the current instruction being executied

wire [31:0]exe_A;

wire [31:0]exe_B;

wire [31:0]exe_op2;

wire [31:0]exe_O;

wire [31:0]exe_extended; // The wire the comes for the sign extender

wire exe_zero;

wire [5:0] exe_alu_op;

wire exe_is_branch;

wire exe_op2_sel;

wire [5:0] exe_shift_amount;

wire [31:0] exe_shift_immed;

wire [31:0] exe_shift_target;

wire [31:0] exe_jump_effective_address;

wire [31:0] exe_branch_effective_address;

wire exe_neg;

wire [1:0]exe_branch_type;

wire exe_branch_taken;

wire [31:0]exe_next_pc;

wire [31:0]exe_branch_next_pc;

wire exe_is_jump;

// Execute control signals

// Instantiate mux's for each of the SREC registers to aid the SREC parser.

mux_2_1_32_bit srec_insn_address_mux(

.line0(pc),

.line1(srec_address),

.select(srec_parse),

.output_line(insn_address)

);

mux_2_1_1_bit srec_insn_rw_mux(

.line0(fetch_rw),

.line1(srec_rw),

.select(srec_parse),

.output_line(insn_rw)

);

mux_2_1_2_bit srec_insn_access_size_mux(

.line0(fetch_access_size),

.line1(srec_access_size),

.select(srec_parse),

.output_line(insn_access_size)

);

// Instantiate the fetch module

fetch fetch(

.clk_in(clk),

.stall_in(stall),

.pc_out(pc),

.rw_out(fetch_rw),

.access_size_out(fetch_access_size)

);

// Instantiate the instruction memory module

memory insn_memory(

.data_out(insn_data_out),

.address(insn_address),

.data_in(srec_data_in), // We can tie the srec_data_in wire to this port since we should never be writing to instruction memory unless we are srec parsing

.write(insn_rw),

.clk(clk),

.access_size(insn_access_size)

);

// Instatiate the IF/ID pipeline register to kep the PC and IR

if_id_pipleline_reg if_id_pipleline_reg(

.clk(clk),

.pc_in(pc),

.ir_in(insn_data_out),

.pc_out(decode_pc),

.ir_out(decode_ir)

);

// Instantiate a mux for selecting which destination to choose

mux_2_1_5_bit dest_reg_mux(

.line0(rt),

.line1(rd),

.select(dest_reg_sel), // TODO: Implement this control singal

.output_line(dest_reg)

);

// Instantiate the register file

reg_file reg_file(

.clk(clk),

.write_enable(reg_file_write_enable),

.source1(rs),

.source2(rt),

.dest(dest_reg),

.destVal(exe_O), // TODO: Change this to be the output from WB stage

.s1val(dec_A),

.s2val(dec_B)

);

// Instantiate the decode module

decode decoder(

.clk(clk),

.stall(stall),

.insn_in(decode_ir),

.pc_in(pc), // TODO: I don't see what this is needed

.rs(rs),

.rt(rt),

.rd(rd),

.sha(sha),

.func(func),

.immed(immed),

.target(target),

.opcode(opcode),

.pc_out(pc_out), // TODO: I don't see why this is needed

.insn_out(insn_out)

);

// Instatiate the ID/IX pipeline register

id_ix_pipleline_reg id_ix_pipleline_reg(

.clk(clk),

.pc_in(decode_pc),

.ir_in(decode_ir),

.A_in(dec_A),

.B_in(dec_B),

.alu_op_in(dec_alu_op),

.is_branch_in(dec_is_branch),

.is_jump_in(dec_is_jump),

.op2_sel_in(dec_op2_sel),

.shift_amount_in(dec_shift_amount),

.branch_type_in(dec_branch_type),

.pc_out(exe_pc),

.ir_out(exe_ir),

.A_out(exe_A),

.B_out(exe_B),

.alu_op_out(exe_alu_op),

.is_branch_out(exe_is_branch),

.is_jump_out(exe_is_jump),

.op2_sel_out(exe_op2_sel),

.shift_amount_out(exe_shift_amount),

.branch_type_out(exe_branch_type)

);

sign_extender sign_extender(

.in_data(exe_ir[15:0]),

.out_data(exe_extended)

);

assign exe_shift_target = exe_ir[25:0];

assign exe_jump_effective_address = (exe_pc & 32'hf0000000) | ((exe_shift_target << 2) & 32'h0fffffff);

assign exe_shift_immed = exe_extended << 2;

assign exe_branch_effective_address = exe_shift_immed + exe_pc;

// Instantiate a 32-bit mux for selecting which operand to provide to op2 of the ALU

mux_2_1_32_bit alu_op2_sel_mux(

.line0(exe_B),

.line1(exe_extended),

.select(exe_op2_sel),

.output_line(exe_op2)

);

alu alu(

.op1(exe_A), // operand 1 (always from rs)

.op2(exe_op2), // operand 2

.operation(exe_alu_op), // The arithmatic operation to perform

.shift_amount(exe_shift_amount), // The number of bits to shift

.result(exe_O), // The arithmatic result based on the operation

.zero(exe_zero), // Indicates if the result of the operation is zero.

.neg(exe_neg)

);

branch_resolve branch_resolve(

.zero(exe_zero),

.neg(exe_neg),

.branch_type(exe_branch_type),

.is_branch(exe_is_branch),

.branch_taken(exe_branch_taken) // Indicates if the branch is taken or not

);

mux_2_1_32_bit branch_next_pc_mux(

.line0(exe_pc),

.line1(exe_branch_effective_address),

.select(exe_branch_taken),

.output_line(exe_branch_next_pc)

);

mux_2_1_32_bit jump_next_pc_mux(

.line0(exe_branch_next_pc),

.line1(exe_jump_effective_address),

.select(exe_is_jump),

.output_line(exe_next_pc)

);

// Control

always @(posedge clk) begin

reg_file_write_enable = 0; // TODO: Change this later

// ----------- Decode Stage Control Signal Logic --------------------------- //

dec_illegal_insn = 0;

dest_reg_sel = 0;

dec_alu_op = 0;

dec_op2_sel = 0;

dec_is_branch = 0;

dec_is_jump = 0;

dec_branch_type = 0;

if (((opcode & 6'b111000)>> 3) == 3'h0) begin

if ((opcode & 6'b000111) == 3'h0) begin

// We are in the SPECIAL Opcode encoding table

// This is an R-type instruction

dest_reg_sel = 0;

if (func == 6'b100000 || func == 6'b100001) // ADD, ADDU

dec_alu_op = 0; // Do an add operation

else if (func == 6'b100010 || func == 6'b100011) // SUB, SUBU

dec_alu_op = 1;

else if (func == 6'b011000 || func == 6'b011001) // MULT, MULTU

dec_alu_op = 2;

else if (func == 6'b011010 || func == 6'b011011) // DIV, DIVU

dec_alu_op = 3;

else if (func == 6'b000000) begin // SLL

dec_alu_op = 4;

dec_shift_amount = sha;

end

else if (func == 6'b000010) begin // SLL

dec_alu_op = 5;

dec_shift_amount = sha;

end

else if (func == 6'b101010 || func == 6'b101011) // SLT, SLTU

dec_alu_op = 6;

else begin

dec_illegal_insn = 1;

end

end else if (((opcode & 6'b000111)  == 3'd2) || ((opcode & 6'b000111) == 3'd3)) begin

// J and JAL instructions

// This is J-type instruction

dest_reg_sel = 0;

dec_is_jump = 1;

if (opcode == 6'b000011) begin

// JAL instruction

dec_illegal_insn = 1;

// TODO: We need to store the return address into reg[31]

end

end else begin

// BEQ, BNE, BLEZ, BGTZ

// This is an I-type instruction

dec_is_branch = 1;

if (opcode == 6'b000100) begin

// BEQ

dec_alu_op = 1; // We want to do a test if the result is zero from a subtract

dec_branch_type = 0;

end else if (opcode == 6'b000101) begin

// BNE

dec_alu_op = 1; // We want to do a test if the result is zero from a subtract

dec_branch_type = 1;

end else if (opcode == 6'b000110) begin

// BLEZ

// TODO: Test that rt has zero in it

dec_illegal_insn = 1;

dec_alu_op = 0;

dec_branch_type = 2;

end else begin

// BGTZ

// TODO: Test that rt has zero in it

dec_illegal_insn = 1;

dec_alu_op = 0;

dec_branch_type = 3;

end

end

end else if (((opcode & 6'b111000)>> 3) == 3'd1) begin

// ADDI, ADDIU, SLTI, SLTIU, ANDI, ORI, XORI, LUI

// This is an I-type instruction

case (opcode)

6'b001000: dec_alu_op = 0;  // ADDI

6'b001001: dec_alu_op = 0;  // ADDIU

6'b001010: dec_alu_op = 6;  // SLTI

6'b001011: dec_alu_op = 6;  // SLTUI

6'b001100: dec_alu_op = 7;  // ANDI

6'b001101: dec_alu_op = 8;  // ORI

6'b001110: dec_alu_op = 9;  // XORI

6'b001111: dec_alu_op = 12; // LUI

endcase

dest_reg_sel = 1;

dec_op2_sel = 1; // We want op2 to be the immediate in the ALU

end else if (((opcode & 6'b111000)>> 3) == 3'd3) begin

// No idea?

dest_reg_sel = 0;

dec_illegal_insn = 1;

end else if (((opcode & 6'b111000)>> 3) == 3'd4) begin

// LB, LH, LWL, LW, LBU, LHU, LWR

// This is an I-type instruction

dest_reg_sel = 1;

dec_op2_sel = 1;

dec_alu_op = 0; // We want to add the value of rs to the immediate

end else if (((opcode & 6'b111000)>> 3) == 3'd5) begin

// SB, SH, SWL, SW, SWR

// This is an I-type instruction

dest_reg_sel = 1;

dec_op2_sel = 1;

dec_alu_op = 0; // We want to add the value of rs to the immediate

end else begin

dec_illegal_insn = 1;

end

end

endmodule

# Processor Test Bench

We did not change anything in our test bench actually but the code is shown here again for reference.

module processor_tb;

reg clk;

reg srec_parse; // enable signal for srec parser.

// Registers, writes, and variables for parser

integer fh = 0; // file handler for input

integer i = 0; // loop variable

integer data_byte = 0; // variable to keep track of what byte we are on.

integer data_offset = 0; // keep track of the offset from the data address to write the next byte.

reg [1:0]nibble_count = 0; // keep track of which nibble is being written

reg [7:0]rec_type; // record type number

reg [7:0] byte_count; //byte count for address, data, and checksum

integer record_code; // A record_code is equivalent to 1 ASCII digit/letter in the .srec file

reg [31:0] rec_address = 'b0; // the address given by the record.

integer highest_address = 0; // highest address written by parser

reg [7:0]rec_data; // A single byte of the data from the record.

reg [7:0]temp; // a temporary byte used for place holding.

reg done = 0; // this will set high when we are done parsing the file.

reg [7:0] file_char = 8'h0A; // Set the initial character from the file read to be the new line character

// Instantiate the processor as the unit under test.

processor processor_uut(.clk(clk), .srec_parse(srec_parse));

initial begin

//---------------------------------------------------------

// Parsing stage of testbench - memory is not valid until

// after the parser has finished!

//---------------------------------------------------------

// Before we begin parsing we want to make sure the fetch module does nothing until the memory has been populated with instructions. We will do this by asserting stall.

processor_uut.stall = 1;

srec_parse = 1;

$monitor("Starting the SREC parser...");

// Open the SREC file to read

fh = $fopen("D:/ECE621_PiplinedProcessor/BubbleSort.srec", "r");

// Start the clock high

clk = 1;

// loop until we set the done bit

while (done == 0) begin

#100; // Delay 1 clock cycle.

// Read the first/next character from the file.

file_char = $fgetc(fh);

if (file_char == 8'hff) begin

done = 1;

file_char = 8'h0A;

end

// Reset the record byte which keeps track of the current byte of the line you are reading in.

record_code = 0;

// Loop until we reach a new line character (new record).

while (file_char != 8'h0A) begin

#50; // Delay 1/2 clock cycle.

highest_address = (rec_address > highest_address)? rec_address: highest_address;

if (record_code == 0) begin

// Clear out all the bit fields.

rec_type = 8'h4;

byte_count = 16'h0;

rec_address = 32'h0;

rec_data = 132'h0;

data_offset = 0;

data_byte = 0;

end else if (record_code == 1) begin

// read the record type.

rec_type[7:0] = atoh(file_char);

end else if (record_code == 2) begin

// read the upper byte of the byte count.

temp = atoh(file_char);

byte_count[7:4] = temp[3:0];

end else if (record_code == 3) begin

// read the lower byte of the byte count.

temp = atoh(file_char);

byte_count[3:0] = temp[3:0];

end else if (record_code > 3) begin

if (rec_type == 1) begin // If record is a 16 bit address.

rec_address[31:16] = 16'h0000;

if (record_code == 4) begin

// read the middle byte of the address.

temp = atoh(file_char);

// remove the upper most nibble since we only have single digits to represent memory addresses

rec_address[15:12] = temp[3:0];

end else if (record_code == 5) begin

temp = atoh(file_char);

rec_address[11:8] = temp[3:0];

end else if (record_code == 6) begin

temp = atoh(file_char);

rec_address[7:4] = temp[3:0];

end else if (record_code == 7) begin

temp = atoh(file_char);

rec_address[3:0] = temp[3:0];

end else begin

// Check to see if we reached the end of the data

if (data_byte < byte_count - 2 - 1) begin

temp = atoh(file_char);

rec_data = rec_data << 4;

rec_data[3:0] = temp[3:0];

nibble_count = nibble_count + 1;

#50;

if (nibble_count > 1) begin

processor_uut.srec_address = rec_address+data_offset;

processor_uut.srec_data_in = rec_data;

processor_uut.srec_access_size = 2'b01;

processor_uut.srec_rw = 1;

#100; // Delay one clock cycle

processor_uut.srec_rw = 0;

// update the data_offest.

data_offset = data_offset + 1;

// reset the nibble count

nibble_count = 0;

data_byte = data_byte + 1;

end

end

end

end

if (rec_type == 2) begin // If record is 24 bit address.

rec_address[31:24] = 8'h00;

if (record_code == 4) begin

temp = atoh(file_char);

rec_address[23:20] = temp[3:0];

end else if (record_code == 5) begin

temp = atoh(file_char);

rec_address[19:16] = temp[3:0];

end else if (record_code == 6) begin

temp = atoh(file_char);

rec_address[15:12] = temp[3:0];

end else if (record_code == 7) begin

temp = atoh(file_char);

rec_address[11:8] = temp[3:0];

end else if (record_code == 8) begin

temp = atoh(file_char);

rec_address[7:4] = temp[3:0];

end else if (record_code == 9) begin

temp = atoh(file_char);

rec_address[3:0] = temp[3:0];

end else begin

// Check to see if we reached the end of the data

if (data_byte < byte_count - 3 - 1) begin

temp = atoh(file_char);

rec_data = rec_data << 4;

rec_data[3:0] = temp[3:0];

nibble_count = nibble_count + 1;

#50;

if (nibble_count > 1) begin

processor_uut.srec_address = rec_address+data_offset;

processor_uut.srec_data_in = rec_data;

processor_uut.srec_access_size = 2'b00;

processor_uut.srec_rw = 1;

#100; // Delay one clock cycle

processor_uut.srec_rw = 0;

// update the data_offest.

data_offset = data_offset + 1;

// reset the nibble count

nibble_count = 0;

data_byte = data_byte + 1;

end

end

end

end

if (rec_type == 3) begin //If record is a 32 bit address.

if (record_code == 4) begin

// read the upper most byte of the address.

temp = atoh(file_char);

// remove the upper most nibble since we only have single digits to represent memory addresses

rec_address[31:28] = temp[3:0];

end else if (record_code == 5) begin

temp = atoh(file_char);

rec_address[27:24] = temp[3:0];

end else if (record_code == 6) begin

temp = atoh(file_char);

rec_address[23:20] = temp[3:0];

end else if (record_code == 7) begin

temp = atoh(file_char);

rec_address[19:16] = temp[3:0];

end else if (record_code == 8) begin

temp = atoh(file_char);

rec_address[15:12] = temp[3:0];

end else if (record_code == 9) begin

temp = atoh(file_char);

rec_address[11:8] = temp[3:0];

end else if (record_code == 10) begin

temp = atoh(file_char);

rec_address[7:4] = temp[3:0];

end else if (record_code == 11) begin

temp = atoh(file_char);

rec_address[3:0] = temp[3:0];

end else begin

// Check to see if we reached the end of the data

if (data_byte < byte_count - 4 - 1) begin

temp = atoh(file_char);

rec_data = rec_data << 4;

rec_data[3:0] = temp[3:0];

nibble_count = nibble_count + 1;

#50;

if (nibble_count > 1) begin

processor_uut.srec_address = rec_address+data_offset;

processor_uut.srec_data_in = rec_data;

processor_uut.srec_access_size = 2'b00;

processor_uut.srec_rw = 1;

#100; // Delay one clock cycle

processor_uut.srec_rw = 0;

// update the data_offest.

data_offset = data_offset + 1;

// reset the nibble count

nibble_count = 0;

data_byte = data_byte + 1;

end

end

end

end

end

#50; // delay 1/2 clock cycle

// increment record_code

record_code = record_code + 1;

// read the next character from the file.

file_char = $fgetc(fh);

end

end

srec_parse = 0;

#100;

// Close up the file

$fclose(fh);

$monitor("Done parsing the SREC file!");

#100;

// ------------------------------------------------------------

// Memory is ready to be used after this point!

// ------------------------------------------------------------

$monitor("Beginning the fetch-decode-execute loop!");

#100;

// Deassert stall just read out the pc, rw, and access size.

processor_uut.stall = 0;

// ------------------------------------------------------------

// This section prints out the decoded instructions

// ------------------------------------------------------------

while (processor_uut.pc <= highest_address - 4) begin

@(posedge clk);

case(processor_uut.opcode)

6'd0:  begin //JR, ADD, ADDU, SUB SUBU, DIV, SLT, SLTU, SLL, SRL, SRA, AND, OR, XOR, NOR, NOP

case(processor_uut.func)

6'd0: begin //SLL, NOP

if (processor_uut.sha == 5'b0) $strobe("%h:    %h    NOP", processor_uut.pc_out, processor_uut.insn_out);

else $strobe("%h:    %h    sll  %h, %h, %h",

processor_uut.pc_out, processor_uut.insn_out, processor_uut.rd, processor_uut.rt, processor_uut.sha);

end

6'd2: begin //SRL

$strobe("%h:    %h    srl  %h, %h, %h",

processor_uut.pc_out, processor_uut.insn_out, processor_uut.rd, processor_uut.rt, processor_uut.sha);

end

6'd3: begin //SRA

$strobe("%h:    %h    sra  %h, %h, %h",

processor_uut.pc_out, processor_uut.insn_out, processor_uut.rd, processor_uut.rt, processor_uut.sha);

end

6'd8: begin //JR

$strobe("%h:    %h    jr  %h",

processor_uut.pc_out, processor_uut.insn_out, processor_uut.rs);

end

6'd26: begin //DIV

$strobe("%h:    %h    div  %h, %h",

processor_uut.pc_out, processor_uut.insn_out, processor_uut.rs, processor_uut.rt);

end

6'd32: begin //ADD

$strobe("%h:    %h    add  %h, %h, %h",

processor_uut.pc_out, processor_uut.insn_out, processor_uut.rd, processor_uut.rs, processor_uut.rt);

end

6'd33: begin //ADDU

$strobe("%h:    %h    addu  %h, %h, %h",

processor_uut.pc_out, processor_uut.insn_out, processor_uut.rd, processor_uut.rs, processor_uut.rt);

end

6'd34: begin //SUB

$strobe("%h:    %h    sub  %h, %h, %h",

processor_uut.pc_out, processor_uut.insn_out, processor_uut.rd, processor_uut.rs, processor_uut.rt);

end

6'd35: begin //SUBU

$strobe("%h:    %h    subu  %h, %h, %h",

processor_uut.pc_out, processor_uut.insn_out, processor_uut.rd, processor_uut.rs, processor_uut.rt);

end

6'd36: begin //AND

$strobe("%h:    %h    and  %h, %h, %h",

processor_uut.pc_out, processor_uut.insn_out, processor_uut.rd, processor_uut.rs, processor_uut.rt);

end

6'd37: begin //OR

$strobe("%h:    %h    or  %h, %h, %h",

processor_uut.pc_out, processor_uut.insn_out, processor_uut.rd, processor_uut.rs, processor_uut.rt);

end

6'd38: begin //XOR

$strobe("%h:    %h    xor  %h, %h, %h",

processor_uut.pc_out, processor_uut.insn_out, processor_uut.rd, processor_uut.rs, processor_uut.rt);

end

6'd39: begin //NOR

$strobe("%h:    %h    nor  %h, %h, %h",

processor_uut.pc_out, processor_uut.insn_out, processor_uut.rd, processor_uut.rs, processor_uut.rt);

end

6'd42: begin //SLT

$strobe("%h:    %h    slt  %h, %h, %h",

processor_uut.pc_out, processor_uut.insn_out, processor_uut.rd, processor_uut.rs, processor_uut.rt);

end

6'd43: begin //SLTU

$strobe("%h:    %h    sltu  %h, %h, %h",

processor_uut.pc_out, processor_uut.insn_out, processor_uut.rd, processor_uut.rs, processor_uut.rt);

end

default: begin

$strobe("ERROR! %h:    %h", processor_uut.pc_out, processor_uut.insn_out);

$stop;

end

endcase

end

6'd1: begin //BLTZ, BGEZ

case(processor_uut.rt)

5'd0: begin //BLTZ

$strobe("%h:    %h    bltz  %h, %h",

processor_uut.pc_out, processor_uut.insn_out, processor_uut.rs, processor_uut.immed);

end

5'd1: begin //BGEZ

$strobe("%h:    %h    bgez  %h, %h",

processor_uut.pc_out, processor_uut.insn_out, processor_uut.rs, processor_uut.immed);

end

default: begin

$strobe("ERROR! %h:    %h", processor_uut.pc_out, processor_uut.insn_out);

$stop;

end

endcase

end

6'd2: begin //J

//display the destination address of the jump, not the offset.

$strobe("%h:    %h    j  %h",

processor_uut.pc_out, processor_uut.insn_out, {processor_uut.pc_out[31-:4], 28'b0}+{processor_uut.target,2'b0});

end

6'd3: begin //JAL

//display the destination address of the jump, not the offset.

$strobe("%h:    %h    jal  %h",

processor_uut.pc_out, processor_uut.insn_out, {processor_uut.pc_out[31-:4], 28'b0}+{processor_uut.target,2'b0});

end

6'd4: begin //BEQ

$strobe("%h:    %h    beq  %h, %h, %h",

processor_uut.pc_out, processor_uut.insn_out, processor_uut.rs, processor_uut.rt, processor_uut.immed);

end

6'd5: begin //BNE

$strobe("%h:    %h    bne  %h, %h, %h",

processor_uut.pc_out, processor_uut.insn_out, processor_uut.rs, processor_uut.rt, processor_uut.immed);

end

6'd6: begin //BLEZ

$strobe("%h:    %h    beq  %h, %h",

processor_uut.pc_out, processor_uut.insn_out, processor_uut.rs, processor_uut.immed);

end

6'd7: begin //BGTZ

$strobe("%h:    %h    bgtz  %h, %h",

processor_uut.pc_out, processor_uut.insn_out, processor_uut.rs, processor_uut.immed);

end

6'd9: begin //ADDIU

$strobe("%h:    %h    addiu  %h, %h, %d",

processor_uut.pc_out, processor_uut.insn_out, processor_uut.rt, processor_uut.rs, processor_uut.immed);

end

6'd10: begin //SLTI

$strobe("%h:    %h    slti  %h, %h, %h",

processor_uut.pc_out, processor_uut.insn_out, processor_uut.rt, processor_uut.rs, processor_uut.immed);

end

6'd13: begin //ORI

$strobe("%h:    %h    ori  %h, %h, %h",

processor_uut.pc_out, processor_uut.insn_out, processor_uut.rt, processor_uut.rs, processor_uut.immed);

end

6'd15: begin //LUI

$strobe("%h:    %h    lui  %h, %h",

processor_uut.pc_out, processor_uut.insn_out, processor_uut.rt, processor_uut.immed);

end

6'd28: begin //MUL

$strobe("%h:    %h    mul  %h, %h, %h",

processor_uut.pc_out, processor_uut.insn_out, processor_uut.rd, processor_uut.rs, processor_uut.rt);

end

6'd32: begin //LB

$strobe("%h:    %h    lb  %h, %d(%h)",

processor_uut.pc_out, processor_uut.insn_out, processor_uut.rt, processor_uut.immed, processor_uut.rs);

end

6'd35: begin //LW

$strobe("%h:    %h    lw  %h, %d(%h)",

processor_uut.pc_out, processor_uut.insn_out, processor_uut.rt, processor_uut.immed, processor_uut.rs);

end

6'd36: begin //LBU

$strobe("%h:    %h    lbu  %h, %d(%h)",

processor_uut.pc_out, processor_uut.insn_out, processor_uut.rt, processor_uut.immed, processor_uut.rs);

end

6'd40: begin //SB

$strobe("%h:    %h    sb  %h, %d(%h)",

processor_uut.pc_out, processor_uut.insn_out, processor_uut.rt, processor_uut.immed, processor_uut.rs);

end

6'd43: begin //SW

$strobe("%h:    %h    sw  %h, %d(%h)",

processor_uut.pc_out, processor_uut.insn_out, processor_uut.rt, processor_uut.immed, processor_uut.rs);

end

default: begin

$strobe("ERROR! %h:    %h", processor_uut.pc_out, processor_uut.insn_out);

$stop;

end

endcase

end

#100;

$stop;

end

always begin

#50 clk = !clk;

end

// A function to convert ASCII upper case letters and digits to their hexadecimal value.

function [7:0]atoh;

input [7:0]aCode;

begin

if (aCode >= 8'h30 && aCode <= 8'h39) begin

atoh = aCode - 8'h30;

end else if (aCode >= 8'h41 && aCode <= 8'h5A) begin

atoh = aCode - 8'h37;

end

end

endfunction

endmodule

# Verification Demonstration

For the verification the setup for us that it is easiest to show is the waveform diagram as the program executes. For readability purposes we have only shown 4 instructions per waveform along with the instruction printout we get from our test bench (this was verified last PD). We show the operands to the ALU, and output from the ALU as decimal to improve readability. The plum colored wave forms are for the fetch stage while the cyan ones are for decode and the green for execute. We executed the bubble sort bench mark in the following diagrams.

# 80020004:    27bdffc8    addiu  1d, 1d, 65480

# 80020008:    afbf0034    sw  1f,    52(1d)

# 8002000c:    afbe0030    sw  1e,    48(1d)

# 80020010:    03a0f021    addu  1e, 1d, 00

# 80020014:    2402000c    addiu  02, 00,    12

# 80020018:    afc20010    sw  02,    16(1e)

# 8002001c:    24020009    addiu  02, 00,     9

# 80020020:    afc20014    sw  02,    20(1e)

# 80020024:    24020004    addiu  02, 00,     4

# 80020028:    afc20018    sw  02,    24(1e)

# 8002002c:    24020063    addiu  02, 00,    99

# 80020030:    afc2001c    sw  02,    28(1e)

# 80020034:    24020078    addiu  02, 00,   120

# 80020038:    afc20020    sw  02,    32(1e)

# 8002003c:    24020001    addiu  02, 00,     1

# 80020040:    afc20024    sw  02,    36(1e)

# 80020044:    24020003    addiu  02, 00,     3

# 80020048:    afc20028    sw  02,    40(1e)

# 8002004c:    2402000a    addiu  02, 00,    10

# 80020050:    afc2002c    sw  02,    44(1e)

# 80020054:    27c20010    addiu  02, 1e,    16

# 80020058:    00402021    addu  04, 02, 00

# 8002005c:    24050008    addiu  05, 00,     8

# 80020060:    0c008020    jal  80020080

# 80020064:    00000000    NOP

# 80020068:    00001021    addu  02, 00, 00

# 8002006c:    03c0e821    addu  1d, 1e, 00

# 80020070:    8fbf0034    lw  1f,    52(1d)

# 80020074:    8fbe0030    lw  1e,    48(1d)

# 80020078:    27bd0038    addiu  1d, 1d,    56

# 8002007c:    03e00008    jr  1f

# 80020080:    00000000    NOP

# 80020084:    27bdffe8    addiu  1d, 1d, 65512

# 80020088:    afbe0014    sw  1e,    20(1d)

# 8002008c:    03a0f021    addu  1e, 1d, 00

# 80020090:    afc40018    sw  04,    24(1e)

# 80020094:    afc5001c    sw  05,    28(1e)

# 80020098:    afc00000    sw  00,     0(1e)

# 8002009c:    0800805f    j  8002017c

# 800200a0:    00000000    NOP

# 800200a4:    24020001    addiu  02, 00,     1

# 800200a8:    afc20004    sw  02,     4(1e)

# 800200ac:    08008055    j  80020154

# 800200b0:    00000000    NOP

# 800200b4:    8fc20004    lw  02,     4(1e)

# 800200b8:    2442ffff    addiu  02, 02, 65535

# 800200bc:    00021080    sll  02, 02, 02

# 800200c0:    8fc30018    lw  03,    24(1e)

# 800200c4:    00621021    addu  02, 03, 02

# 800200c8:    8c430000    lw  03,     0(02)

# 800200cc:    8fc20004    lw  02,     4(1e)

# 800200d0:    00021080    sll  02, 02, 02

# 800200d4:    8fc40018    lw  04,    24(1e)

# 800200d8:    00821021    addu  02, 04, 02

# 800200dc:    8c420000    lw  02,     0(02)

# 800200e0:    0043102a    slt  02, 02, 03

# 800200e4:    10400019    beq  02, 00, 0019

# 800200e8:    00000000    NOP

# 800200ec:    8fc20004    lw  02,     4(1e)

# 800200f0:    2442ffff    addiu  02, 02, 65535

# 800200f4:    00021080    sll  02, 02, 02

# 800200f8:    8fc30018    lw  03,    24(1e)

# 800200fc:    00621021    addu  02, 03, 02

# 80020100:    8c420000    lw  02,     0(02)

# 80020104:    afc20008    sw  02,     8(1e)

# 80020108:    8fc20004    lw  02,     4(1e)

# 8002010c:    2442ffff    addiu  02, 02, 65535

# 80020110:    00021080    sll  02, 02, 02

# 80020114:    8fc30018    lw  03,    24(1e)

# 80020118:    00621021    addu  02, 03, 02

# 8002011c:    8fc30004    lw  03,     4(1e)

# 80020120:    00031880    sll  03, 03, 02

# 80020124:    8fc40018    lw  04,    24(1e)

# 80020128:    00831821    addu  03, 04, 03

# 8002012c:    8c630000    lw  03,     0(03)

# 80020130:    ac430000    sw  03,     0(02)

# 80020134:    8fc20004    lw  02,     4(1e)

# 80020138:    00021080    sll  02, 02, 02

# 8002013c:    8fc30018    lw  03,    24(1e)

# 80020140:    00621021    addu  02, 03, 02

# 80020144:    8fc30008    lw  03,     8(1e)

# 80020148:    ac430000    sw  03,     0(02)

# 8002014c:    8fc20004    lw  02,     4(1e)

# 80020150:    24420001    addiu  02, 02,     1

# 80020150:    24420001    addiu  02, 02,     1

# 80020154:    afc20004    sw  02,     4(1e)

# 80020158:    8fc3001c    lw  03,    28(1e)

# 8002015c:    8fc20000    lw  02,     0(1e)

# 80020164:    8fc20004    lw  02,     4(1e)

# 80020168:    0043102a    slt  02, 02, 03

# 8002016c:    1440ffd1    bne  02, 00, ffd1

# 80020170:    00000000    NOP

# 80020174:    8fc20000    lw  02,     0(1e)

# 80020178:    24420001    addiu  02, 02,     1

# 8002017c:    afc20000    sw  02,     0(1e)

# 80020180:    8fc30000    lw  03,     0(1e)

# 80020184:    8fc2001c    lw  02,    28(1e)

# 80020188:    0062102a    slt  02, 03, 02

# 8002018c:    1440ffc5    bne  02, 00, ffc5

# 80020190:    00000000    NOP

# 80020194:    03c0e821    addu  1d, 1e, 00

# 80020198:    8fbe0014    lw  1e,    20(1d)

# 8002019c:    27bd0018    addiu  1d, 1d,    24